All fasteners in metric (mm).

(50)	PIN, TAPER, EXT THREAD, M6 x 1 x 32;
CARBON STEEL, ASTM A 29, GR 1045, 
UNS G10450,
TYPE C, GRADE 2, CLASS B,
MAX THREAD LENGTH = 2.9,
TAPER = 1:50, TOL =  +/-10’ OF ANGLE
OAL = 31.5-32.5

(50)	PIN, TAPER, EXT THREAD, M8 x 1.25 x 50;
CARBON STEEL, ASTM A 29, GR 1045, 
UNS G10450,
TYPE C, GRADE 2, CLASS B,
MAX THREAD LENGTH = 2.9,
TAPER = 1:50, TOL =  +/-10’ OF ANGLE
OAL = 49.5-50.5

Will Hyde

MK110 ISEA GWSRP Technician

Medium Caliber Guns Branch – W83

NSWC Indian Head Division - Detachment Picatinny

Bldg 61N, Picatinny Arsenal, NJ 07806-5000

Office: (973) 724-0873
Cell:     (971) 201-1015

william.d.hyde20.civ@us.navy.mil